Test Cases: Automated Glacier Delineation of Chhota Shigri (GeoAI)

IoU = Intersection over Union | HMA = High Mountain Asia | RGI = Randolph Glacier Inventory | CI = 95 % confidence interval

| Test Case ID | Test Case Objective | Pre requisite | Steps | Input Data | Expected Output | Actual Output | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TC_01 | Locate study glacier from inventory automatically | Internet access to GLIMS WFS | 1. Run src.data.inventory 2. Read RGI6/RGI7 layers 3. Build ROI and grid | Region: lon 77.40–77.75, lat 32.10–32.40 | Chhota Shigri found; RGI area ≈ 16.8 km² | Found RGI60-14.15990; area 16.772 km²; grid 1434 × 1254 px | PASS |
| TC_02 | Reject scenes with cloud or fresh snow over the glacier | Planetary Computer and Google bucket access | 1. Search Sep–Oct 2023 2. Screen cloud over glacier 3. Score off-glacier snow | 2023 Sentinel-2 scenes | Scene with fresh snow rejected even if scene cloud is low | 2023-10-06 (0.06 % cloud, 49 % fresh snow) rejected; 2023-09-11 (0.5 % glacier cloud) chosen | PASS |
| TC_03 | Convert Sentinel-2 L1C to correct TOA reflectance | L1C metadata readable | 1. Read MTD_MSIL1C.xml 2. Apply offset 3. Divide by 10000 | Baseline 05.09 | Offset −1000 applied for baseline ≥ 04.00 | RADIO_ADD_OFFSET = −1000 read and applied | PASS |
| TC_04 | Load pretrained GlaViTU and run it | TensorFlow 2.15 environment | 1. Build model 2. Load weights 3. Predict one patch | glavitu_global_weights.h5 384 × 384 patch | Model loads; output 384 × 384 × 2 | 10,032,068 parameters; output (384, 384, 2) | PASS |
| TC_05 | Run inference on Apple M4 GPU | tensorflow-metal installed | 1. Set device = GPU 2. Predict full glacier image | 2023 scene, 8 channels | Prediction completes | Crashed with SIGBUS (exit 138); CPU used instead (30 s per scene) | FAIL |
| TC_06 | First automatic glacier boundary (no manual tracing) | TC_02 – TC_04 passed | 1. Run inference 2. Threshold 0.5 3. Clean mask 4. Polygonise 5. Compute area | 2023-09-11 scene + DEM | Valid polygon and area produced automatically | Area 15.10 km² (M0); IoU 0.839 vs 2002 RGI outline | PASS |
| TC_07 | Co-register images to < 0.5 pixel | Scenes downloaded | 1. Phase correlation on stable rock 2. Shift if > 0.25 px 3. Re-measure | 16 analysis scenes vs 2023-09-11 | All residual shifts < 0.5 px (< 5 m) | Worst 0.67 px → 0.11 px after correction; all years ≤ 0.16 px (≤ 1.6 m) | PASS |
| TC_08 | Check no test leakage (site not in benchmark) | Benchmark tiles downloaded | 1. Read tile bounds 2. Check distance to Chhota Shigri | 277 HMA tiles | 0 tiles within 0.05° of Chhota Shigri | 0 tiles; nearest about 0.6° away | PASS |
| TC_09 | HMA fine-tuning improves accuracy | M0 and M1 weights | 1. Predict 57 held-out test tiles 2. Compute pooled IoU 3. Bootstrap CI | HMA test split (57 tiles) | M1 IoU > M0 IoU, CI above 0 | M0 0.678 → M1 0.748 (+0.070, CI 0.040–0.107) | PASS |
| TC_10 | Generate pseudo-labels with raw thresholds | M1 predictions for even years | 1. P ≥ 0.95 → glacier 2. P ≤ 0.05 → background | 15 scenes (2018–2024) | Labelled pixels > 0 | 0 pixels labelled (raw output only spans 0.24–0.89) | FAIL |
| TC_11 | Pseudo-labels after probability calibration | Isotonic calibrator fitted | 1. Calibrate on HMA val tiles 2. Apply 0.95 / 0.05 3. Check agreement | Same 15 scenes | Calibration error drops; labels produced | ECE 0.239 → 0.011; 292,012 glacier and 881,732 background pixels | PASS |
| TC_12 | Site self-training (M2) does not forget HMA | M2 trained | 1. Predict 57 HMA test tiles 2. Compare with M1 | HMA test split | M2 IoU ≥ M1 IoU (0.748) | M2 IoU 0.779 (+0.031 vs M1) | PASS |
| TC_13 | DEM improves segmentation | Equal-budget models trained | 1. Train optical-only and optical + DEM 2. Test on 22 tiles | 22 SAR-subset test tiles | Optical + DEM > optical | 0.676 → 0.824 (+0.149, CI 0.078–0.241) | PASS |
| TC_14 | SAR (Sentinel-1) improves segmentation | M3 trained | 1. Add VV backscatter branch 2. Train same budget 3. Test | 22 SAR test tiles | Optical + DEM + SAR > optical + DEM | 0.808 vs 0.824 (−0.016); no gain at short budget | FAIL |
| TC_15 | Detect terminus retreat automatically | Ensemble masks for all years | 1. Auto centerline 2. Find terminus each year 3. Measure change across 5 models | 2021 and 2025 masks | Retreat > measurement error (≈ 14 m) in all models | 70–160 m retreat in all 5 models (≈ 20–28 m/yr) | PASS |
| TC_16 | Detect significant area loss 2017–2025 | Areas and uncertainties computed | 1. Weighted trend over 5 years 2. Check 95 % CI | M2 areas 15.00 → 14.30 km² | Trend CI excludes 0 | −0.159 km²/yr, CI −0.402 to +0.084 (includes 0) | FAIL |
| TC_17 | Generate year-of-loss map | Masks for all 5 years | 1. Stack masks 2. Assign loss year only if loss persists | M2 masks 2017–2025 | Raster with loss years and areas | Persistent loss 0.94 km²; unstable 1.74 km² | PASS |
| TC_18 | Spatial pattern of loss is detected | Year-of-loss map | 1. Loss by elevation, slope, distance 2. Hotspots (Gi*) 3. Spatial-block CV | M2 loss map + DEM | Clear pattern; CV AUC > 0.7 | 42 % loss within 250 m of snout; AUC 0.94 | PASS |
| TC_19 | Notebooks run end to end | Outputs generated | 1. Execute all 7 notebooks | notebooks/01–07 | 7/7 run without error | 7/7 executed successfully | PASS |
| TC_20 | Demo opens offline | demo/ folder | 1. Double-click index.html 2. Load from disk | Local file, no internet | Map, boundaries and controls render | Rendered correctly from file:// in Chrome | PASS |

Summary: 20 test cases, 16 PASS, 4 FAIL. TC_05 and TC_10 were resolved by the fixes tested in TC_06 (CPU inference) and TC_11 (calibration). TC_14 and TC_16 are scientific findings: no SAR benefit at the short training budget, and area loss over 2017–2025 is not yet statistically significant.